/**

\page tutorial-install-crosscompiling-naoqi Tutorial: Cross-compilation for NAOqi OS from Ubuntu host

\tableofcontents

This tutorial summarizes how to cross-compile ViSP from source using NAOqi atom cross toolchain provided by SoftBank Robotics for Nao, Romeo or Pepper robots. The aim of this cross-compilation is then to use the cross-build resulting ViSP libraries on these robots.

This tutorial was tested on an Ubuntu 14.04 LTS host computer with:

- Cross Toolchain 2.3.1 Linux 64 (\c ctc-linux32-atom-2.3.1.23) on Romeo robot

- Cross Toolchain 2.4.3 Linux 64 (\c ctc-linux64-atom-2.4.3.28) on Pepper robot

\section cross_naoqi_toolchain Install cross toolchain

Download the cross-toolchain that is compatible with your robot from <a href="https://developer.softbankrobotics.com/">https://developer.softbankrobotics.com/</a>. Depending on the robot the version of the cross toolchain might change.

In this tutorial we will illustrate how to build ViSP for Pepper with \c ctc-linux64-atom-2.4.3.28 cross toolchain.

Create a workspace that contains the cross toolchain:

\code

$ mkdir $HOME/softbank

\endcode

and unzip the toolchain:

\code

$ cd $HOME/softbank

$ unzip ctc-linux64-atom-2.4.3.28.zip

\endcode

\section cross_naoqi_visp_src Get ViSP source code

Cross-compilation capabilities are fully supported since ViSP 3.0.1 release.

Create first a workspace dedicated to ViSP:

\code

$ mkdir $HOME/soft

\endcode

There are different ways to get ViSP source code in this workspace:

- You can download a ViSP sources <a href="https://visp.inria.fr/download#snapshot">daily snapshot</a>. Once downloaded, uncompress the file using

\code

$ tar xvzf visp-snapshot-yyyy-mm-dd.tar.gz --directory $HOME/soft

\endcode

- Or you get the cutting-edge ViSP from <a href="https://github.com/lagadic/visp">GitHub repository</a> using the following command

\code

$ cd $HOME/soft

$ git clone https://github.com/lagadic/visp.git

\endcode

- Or you can download the <a href="http://visp.inria.fr/download">latest release</a> (should be at least 3.0.1) as a zip or a tarball. Once downloaded, uncompress the file using either

\code

$ tar xvzf visp-x.y.z.tar.gz --directory $HOME/soft

\endcode

or

\code

$ unzip visp-x.y.z.zip -d $HOME/soft

\endcode

We suppose now that ViSP source is in a directory denoted \c \<source_dir\>, for example \c $HOME/soft/visp

\section cross_naoqi_visp_config Cross-compiling ViSP from source

- Create first a directory denoted \c \<binary_dir\> where you want to cross-compile ViSP. This directory will contain generated Makefiles, object files, and output libraries and binaries that could be later used on the Raspberry Pi.

\code

$ mkdir $HOME/soft/visp-build-ctc-linux64-atom-2.4.3.28

\endcode

- Enter \c \<binary_dir\> and configure the build:

\code

$ cd $HOME/soft/visp-build-ctc-linux64-atom-2.4.3.28

$ cmake -DCMAKE_TOOLCHAIN_FILE=$HOME/softbank/ctc-linux64-atom-2.4.3.28/toolchain.cmake ../visp

\endcode

\note To speed up the cross-compilation build step, you can turn off the build of the demos, examples, tests and tutorials using the following command:

\code

$ cmake -DCMAKE_TOOLCHAIN_FILE=$HOME/softbank/ctc-linux64-atom-2.4.3.28/toolchain.cmake ../visp -DBUILD_DEMOS=OFF -DBUILD_EXAMPLES=OFF -DBUILD_TESTS=OFF -DBUILD_TUTORIALS=OFF -DQI_WITH_TESTS=OFF

\endcode

- At this step, the following third-parties should be found: \c OpenCV, \c v4l2, \c libjpeg, \c libpng, \c pthread, \c zbar. This could be checked in \c ViSP-third-party.txt file where the you should find something similar to:

\code

$ more ViSP-third-party.txt

OpenCV                      : yes

Video For Linux Two         : yes

libjpeg                     : yes

libpng                      : yes

pthread                     : yes

zbar                        : yes

\endcode

\note \c zbar third-party is available with Cross Toolchain 2.3.1 Linux 64 (\c ctc-linux32-atom-2.3.1.23). This library is not present in Cross Toolchain 2.4.3 Linux 64 (\c ctc-linux64-atom-2.4.3.28).

- Cross-compile ViSP

\code

$ make -j4 install

\endcode

The resulting installation is available in \c $HOME/soft/visp-build-ctc-linux64-atom-2.4.3.28.

\section cross_naoqi_visp_install Install ViSP on NAOqi OS

The result of the cross-compilation could then be installed on Nao, Romeo or Pepper robot running NAOqi OS.

- Copy the cross-build installation to Nao, Romeo or Pepper target:

\code

$ tar cvzf install.tar.gz install

$ scp -r install.tar.gz nao@<your_robot_ip>:

\endcode

- Install ViSP:

First move the libraries that were cross-build to a more friend location like \c $HOME/visp.

\code

$ ssh nao@<your_robot_ip>

nao~ $ tar xvzf ~/install.tar.gz

nao~ $ mv install visp

nao~ $ rm ~/install.tar.gz

\endcode

- Add to \c LD_LIBRARY_PATH environment var the path to ViSP libraries:

\code

nao~ $ export LD_LIBRARY_PATH=$LD_LIBRARY_PATH:$HOME/visp/lib

\endcode

\section cross_naoqi_next Next tutorial

You are now ready to see the next \ref tutorial-getting-started-naoqi that explains how to use ViSP as a 3rd party to build your own project on NAOqi OS for Nao, Romeo or Pepper robots.

*/